; ===================================================================

; STEP 1: Sentaurus Structure Editor (SDE) - Advanced Diode

; ===================================================================

; --- Domain Definition (Width: 2 um, Depth: 10 um) ---

(sdegeo:create-rectangle (position 0 0 0) (position 2 10 0) "Silicon" "R.Substrate")

; --- Electrodes ---

(sdegeo:define-contact-set "Anode"   4 (color:rgb 1 0 0) "##")

(sdegeo:define-contact-set "Cathode" 4 (color:rgb 0 0 1) "##")

; Assign 2D Contacts to Edges

(sdegeo:define-2d-contact (find-edge-id (position 1 0 0)) "Anode")

(sdegeo:define-2d-contact (find-edge-id (position 1 10 0)) "Cathode")

; --- Refinement Windows for Doping Placement ---

(sdedr:define-refeval-window "Win_AnodeDoping" "Rectangle" (position 0 0 0) (position 2 1.0 0))

(sdedr:define-refeval-window "Win_CathodeDoping" "Rectangle" (position 0 9 0) (position 2 10 0))

; --- Doping Profiles ---

; 1. N-Type Substrate Background Doping (Nd = 2e15 cm^-3)

(sdedr:define-constant-profile "Const.Substrate" "PhosphorusActiveConcentration" 2e15)

(sdedr:define-constant-profile-region "Placement_Substrate" "Const.Substrate" "R.Substrate")

; 2. P+ Anode Top Gaussian Implant (Na = 1e19 cm^-3, Junction ~ 0.8 um)

(sdedr:define-gaussian-profile "Gauss.Anode" "BoronActiveConcentration"

"PeakPos" 0 "PeakVal" 1e19 "ValueAtDepth" 2e15 "Depth" 0.8 "Gauss" "Factor" 0.8)

(sdedr:define-analytical-profile-placement "Placement_Anode"

"Gauss.Anode" "Win_AnodeDoping" "Positive" "NoReplace" "Eval")

; 3. N+ Cathode Bottom Gaussian Implant (Nd = 1e20 cm^-3, Depth ~ 0.5 um from edge)

(sdedr:define-gaussian-profile "Gauss.Cathode" "PhosphorusActiveConcentration"

"PeakPos" 10 "PeakVal" 1e20 "ValueAtDepth" 2e15 "Depth" 9.5 "Gauss" "Factor" 0.8)

(sdedr:define-analytical-profile-placement "Placement_Cathode"

"Gauss.Cathode" "Win_CathodeDoping" "Negative" "NoReplace" "Eval")

; --- Mesh Refinement Strategy ---

; Refinement Windows

(sdedr:define-refeval-window "Win_Global" "Rectangle" (position 0 0 0) (position 2 10 0))

(sdedr:define-refeval-window "Win_Junction" "Rectangle" (position 0 0.3 0) (position 2 2.0 0))

(sdedr:define-refeval-window "Win_CathodeMesh" "Rectangle" (position 0 8.5 0) (position 2 10.0 0))

; Refinement Definitions & Placements

(sdedr:define-refinement-size "RefDef_Global" 0.5 0.5 0 0.2 0.2 0)

(sdedr:define-refinement-placement "Place_Global" "RefDef_Global" "Win_Global")

(sdedr:define-refinement-size "RefDef_Junction" 0.05 0.005 0 0.01 0.001 0)

(sdedr:define-refinement-placement "Place_Junction" "RefDef_Junction" "Win_Junction")

(sdedr:define-refinement-size "RefDef_Cathode" 0.1 0.01 0 0.02 0.005 0)

(sdedr:define-refinement-placement "Place_Cathode" "RefDef_Cathode" "Win_CathodeMesh")

; Execute Mesh Generator to Output n1_msh.tdr

(sde:build-mesh "snmesh" "" "n1_msh")

# ===================================================================

# STEP 2: Sentaurus Device Command File (Complete Characterization)

# ===================================================================

File {

Grid    = "n1_msh.tdr"

Output  = "n1_des.tdr"

Plot    = "n1_des.plt"

Current = "n1_des.plt"

Param   = "models.par"

}

Electrode {

{ Name="Anode"   Voltage=0.0 }

{ Name="Cathode" Voltage=0.0 }

}

Thermode {

{ Name="Anode"   Temperature=300 }

{ Name="Cathode" Temperature=300 }

}

Physics {

Temperature = 300

Mobility( DopingDependence HighFieldSaturation Enormal )

Recombination( SRH(DopingDependence) Auger Avalanche(Lackner) )

EffectiveIntrinsicDensity( OldSlotboom )

Thermodynamic

}

Plot {

eDensity hDensity SpaceCharge

eCurrent hCurrent CurrentPotential

ElectricField ElectrostaticPotential

DonorConcentration AcceptorConcentration

eQuasiFermiPotential hQuasiFermiPotential

BandGap SRH Auger AvalancheGeneration

Temperature

}

Math {

Extrapolate

Derivatives

Iterations = 50

RelErrControl

Digits = 5

Method = blocked

NumberOfThreads = 1

BreakCriteria {

Current( Contact = "Cathode" AbsVal = 1.0e-4 )

}

}

Solve {

Poisson

Coupled { Poisson Electron Hole }

Quasistationary(

InitialStep=0.01 Increment=1.5 MaxStep=0.1 MinStep=1e-5

Goal { Name="Anode" Voltage=0.05 }

){

Coupled { Poisson Electron Hole Temperature }

}

NewCurrentFile="Forward_IV_"

Quasistationary(

InitialStep=1e-3 Increment=1.2 MaxStep=0.02 MinStep=1e-7

Goal { Name="Anode" Voltage=1.5 }

){

Coupled { Poisson Electron Hole Temperature }

}

Quasistationary(

InitialStep=0.1 Increment=1.5 MaxStep=0.5 MinStep=1e-4

Goal { Name="Anode" Voltage=0.0 }

){

Coupled { Poisson Electron Hole Temperature }

}

NewCurrentFile="Breakdown_IV_"

Quasistationary(

InitialStep=1e-2 Increment=1.3 MaxStep=2.0 MinStep=1e-6

Goal { Name="Cathode" Voltage=200.0 }

){

Coupled { Poisson Electron Hole Temperature }

}

Quasistationary(

InitialStep=0.1 Increment=1.5 MaxStep=0.5 MinStep=1e-4

Goal { Name="Cathode" Voltage=0.0 }

){

Coupled { Poisson Electron Hole }

}

NewCurrentFile="Reverse_CV_Sweep_"

Quasistationary(

InitialStep=0.05 Increment=1.2 MaxStep=0.2 MinStep=1e-4

Goal { Name="Cathode" Voltage=10.0 }

){

Coupled { Poisson Electron Hole }

}

Quasistationary(

InitialStep=0.1 Increment=1.5 MaxStep=0.5 MinStep=1e-4

Goal { Name="Cathode" Voltage=0.0 }

){

Coupled { Poisson Electron Hole }

}

NewCurrentFile="Transient_RR_Init_"

Quasistationary(

InitialStep=0.1 Increment=1.2 MaxStep=0.5 MinStep=1e-4

Goal { Name="Anode" Voltage=1.0 }

){

Coupled { Poisson Electron Hole }

}

NewCurrentFile="Transient_RR_"

Transient(

InitialTime=0 FinalTime=100e-9

InitialStep=1e-12 MaxStep=1e-9 MinStep=1e-14

){

Coupled { Poisson Electron Hole }

}

}

# ===================================================================

# Comprehensive Sentaurus Visual (svisual) Automation Script

# Automatically plots Mesh Structure, IV Curves, CV Sweep, & Transients

# ===================================================================

# 1. Visualize the Device Structure and Mesh

load_file "n1_msh.tdr"

create_plot -type mesh -name "Device_Structure"

# 2. Visualize Forward I-V Characteristics

load_file "Forward_IV_n1_des.plt"

create_plot -type xy -name "Forward_IV"

add_curve -plot "Forward_IV" -dataset "Forward_IV_n1_des" -x "Anode OuterVoltage" -y "Anode TotalCurrent"

set_axis_prop -plot "Forward_IV" -axis x -title "Anode Voltage (V)"

set_axis_prop -plot "Forward_IV" -axis y -title "Anode Total Current (A)"

# Optional: Uncomment the next line to view current on a logarithmic scale

# set_axis_prop -plot "Forward_IV" -axis y -scale log

# 3. Visualize Reverse Breakdown I-V Characteristics

load_file "Breakdown_IV_n1_des.plt"

create_plot -type xy -name "Breakdown_IV"

add_curve -plot "Breakdown_IV" -dataset "Breakdown_IV_n1_des" -x "Cathode OuterVoltage" -y "Cathode TotalCurrent"

set_axis_prop -plot "Breakdown_IV" -axis x -title "Cathode Voltage (V)"

set_axis_prop -plot "Breakdown_IV" -axis y -title "Cathode Total Current (A)"

# 4. Visualize Reverse C-V Sweep (Charge vs Voltage)

load_file "Reverse_CV_Sweep_n1_des.plt"

create_plot -type xy -name "Reverse_CV"

add_curve -plot "Reverse_CV" -dataset "Reverse_CV_Sweep_n1_des" -x "Cathode OuterVoltage" -y "Cathode Charge"

set_axis_prop -plot "Reverse_CV" -axis x -title "Cathode Voltage (V)"

set_axis_prop -plot "Reverse_CV" -axis y -title "Cathode Charge (C)"

# 5. Visualize Initial Transient Recovery Preparation

load_file "Transient_RR_Init_n1_des.plt"

create_plot -type xy -name "Transient_Init"

add_curve -plot "Transient_Init" -dataset "Transient_RR_Init_n1_des" -x "time" -y "Anode TotalCurrent"

set_axis_prop -plot "Transient_Init" -axis x -title "Time (s)"

set_axis_prop -plot "Transient_Init" -axis y -title "Anode Total Current (A)"

# 6. Visualize Final Transient Reverse Recovery Response

load_file "Transient_RR_n1_des.plt"

create_plot -type xy -name "Transient_Response"

add_curve -plot "Transient_Response" -dataset "Transient_RR_n1_des" -x "time" -y "Anode TotalCurrent"

set_axis_prop -plot "Transient_Response" -axis x -title "Time (s)"

set_axis_prop -plot "Transient_Response" -axis y -title "Anode Total Current (A)"